# ANEXA 1

# Lucrarea nr.FLS 2015/20.03.2012

MUNICIPIUL SFANTU GHEORGHE

REABILITARE ILUMINAT CARTIERUL OLTULUI

ANTEMASURATOARE

STRADA NICOLAE COLAN

A119 - Unscrewed zincked steel piping d2 ½					25,5 m

Electrod teava zincata d2 ½

1buc cu L=1,5m la fiecare stalp  => 1,5x17 st= 25,5m

--------------------------------------------

Total = 25,5 m

A102 - Acyaby energy cable 0,6/1 kV, 3x25+16					580 m

Cablu energie  ACYABY 3x25+16  0,6/1 kV

Lungime tronson cu stalpi h= 9 m = 510 m

Lungime tronson  alimentare din PT 139 = 70 m

--------------------------------------------

Total = 580 m

A101 - Al – Zn plate connection bracelet						17 set

Set fixare 1 set / stalp

--------------------------------------------

Pentru 17 stalpi => 17 seturi

A103 - Cable end cover								68 buc

Cap terminal ACYABY 3x25+16 mmp = 2 buc/ cutie legaturi

Cap terminal CYY 3x1,5 mmp = 2 buc/ stalp

--------------------------------------------

Total / stalp =4 buc ,  pentru  stalpi => 17x4= 68 buc

A117 - Rigid pvc piping type g 110 x 8,2						86 m

TUB PVC tip G 110x8,2

Subtraversare cale rutiera 1x2x13 m = 26

Subtraversare cale rutiera 1x2x14 m = 28

Subtraversare cale rutiera 1x2x16 m = 32

--------------------------------------------

Total = 86 m

L.100.N - Luminaire for 100 W high – pressure sodium lamp  					21 buc

Corp de iluminat 100W

1 corp / stalp cu un brat => 1x13buc = 13 buc

2 corp / stalp cu 2 brate => 2x4buc = 8 buc

--------------------------------------------

Total =  21 buc

L.P.09.1 - Lighting pole made of galvanized steel, 9 m height, with one 1.5 m curved arm		13 buc

L.P.09.2 - Lighting pole made of galvanized steel, 9 m height, with two 1.5 m curved arms		4 buc

Stalp de iluminat cu h= 9, cu 1,2 brate curbate de 1,5m

L ef. traseu/ dist. Intre stalpi = 510 m / 30 m = 17=> 17 stalpi

-------------------------------------------

Total =  17 buc.

515 - Distribution panel type FB								1 buc

Cutie distributie echipata

Alimentare retea noua str. Nicolae Colan din PT 139 = 1buc

-----------------------------------------

Total =  1 buc.

Proiectant:

SC Flash Lighting Services SA