# Lucrarea nr.FLS 2017/20.07.2012

MUNICIPIUL SFANTU GHEORGHE

REABILITARE ILUMINAT PUBLIC PARCAREA STRADA ROMULUS CIOFLEC

ANTEMASURATOARE

A119 - Unscrewed zincked steel piping d2 ½					13,5 m

Electrod teava zincata d2 ½

1buc cu L=1,5m la fiecare stalp  => 1,5x9 st= 13,5m

--------------------------------------------

Total = 13.5m

A102 - Acyaby energy cable 0,6/1 kV, 3x25+16					340 m

Cablu energie  ACYABY 3x25+16  0,6/1 kV

Lungime tronson cu stalpi h= 9 m = 280 m

Lungime tronson  alimentare din LEA torsadat = 60 m

--------------------------------------------

Total = 340 m

A101 - Al – Zn plate connection bracelet						9 set

Set fixare 1 set / stalp

--------------------------------------------

Pentru 9 stalpi => 9 seturi

A103 - Cable end cover								36 buc

Cap terminal ACYABY 3x25+16 mmp = 2 buc/ cutie legaturi

Cap terminal CYY 3x1,5 mmp = 2 buc/ stalp

--------------------------------------------

Total / stalp =9 buc ,  pentru  stalpi => 9x4= 36 buc

A117 - Rigid pvc piping type g 110 x 8,2						30 m

TUB PVC tip G 110x8,2

Subtraversare cale rutiera 1x2x9 m = 18

Subtraversare cale rutiera 1x2x6 m = 12

--------------------------------------------

Total = 30 m

L.100.N - Luminaire for 100 W high – pressure sodium lamp  				11 buc

Corp de iluminat 100W

1 corp / stalp cu un brat => 1x7buc = 7 buc

2 corp / stalp cu 2 brate => 2x2buc = 4 buc

--------------------------------------------

Total =  11 buc

L.P.09.1 - Lighting pole made of galvanized steel, 9 m height, with one 1.5 m curved arm		7 buc

L.P.09.2 - Lighting pole made of galvanized steel, 9 m height, with two 1.5 m curved arms		2 buc

Stalp de iluminat cu h= 9, cu 1 si 2 brate curbate de 1,5m

L ef. traseu/ dist. Intre stalpi = 280 m / 30 m =9,33  => 9 stalpi

-------------------------------------------

Total =  9 buc.

515  - Distribution panel type FB								1 buc

Cutie distributie echipata

Alimentare retea noua parcare str. Romulus Cioflec din LEA = 1buc

-----------------------------------------

Total =  1 buc.

Proiectant:

SC Flash Lighting Services SA